Department of CSE

Course Code

CSE406

Course Title

Internet of Things

Lab 03

Section : 01 Semester : Summer2025

Submitted By

| Name | ID |
| --- | --- |
| Seendid Saleh Kabir | 2022-1-60-122 |

Submitted To

Dr. Raihan Ul Islam

Associate Professor

Department of Computer Science and Engineering

East West University

Lab HTTP

ESP8266 ↔ Flask REST – Connectivity Test Report

Goal

Verify that a single NodeMCU (ESP8266) can periodically call a REST endpoint hosted on a PC over the local network and receive JSON responses.

Setup

- Client: NodeMCU 1.0 (ESP-12E) running Arduino sketch (HTTPClient + WiFiMulti).

  - SSID: “Seendid Wifi 2G” (2.4 GHz).

  - Request interval: 10 s (delay(10000)).

- Server: Python Flask app listening on 0.0.0.0:5000 with route /rest.

  - GET → {"message":"GET request received"}

  - POST → echoes JSON

- Network: Same LAN/subnet.

  - Server IP: 192.168.0.103

  - ESP IP: 192.168.0.108

Evidence & Observations

- ESP Serial Monitor (sample):

- WiFi connected

- IP address:

- 192.168.0.108

- [HTTP] begin...

- [HTTP] GET... code: 200

- {"message":"GET request received"}

Repeats every ~10 s.

- Flask Console (sample):

- 192.168.0.108 - - [22/Aug/2025 20:43:44] "GET /rest HTTP/1.1" 200 -

- 192.168.0.108 - - [22/Aug/2025 20:43:54] "GET /rest HTTP/1.1" 200 -

- 192.168.0.108 - - [22/Aug/2025 20:44:04] "GET /rest HTTP/1.1" 200 -

- Timing: Timestamps show requests at ~10-second intervals (e.g., :44, :54, :04, :14).
 Effective rate ≈ 6 requests/min.

- Status: All responses HTTP 200 with expected JSON payload. Connectivity is stable.

Conclusion

The system works as intended with one NodeMCU acting as an HTTP client and a PC-hosted Flask server responding on /rest. The earlier connection issue was due to attempting a 5 GHz SSID; switching to 2.4 GHz resolved it.

VScode Output:

Arduino Output:

Lab COAP
CoAP Light Control — Final Verification Report (ESP8266 + aiocoap)

1) Objective

Validate end-to-end CoAP control of the NodeMCU (ESP8266) “light” endpoint for both commands:

- "1" → Turn ON

- "0" → Turn OFF

2) System Setup

- Device: NodeMCU (ESP8266)

- Firmware (server): coap-simple with endpoint /light (code you shared)

  - Parses payload as text ("1"/"0")

  - Drives active-low on-board LED

  - Replies with current state ("1" when ON, "0" when OFF) and 2.04 Changed

- Wi-Fi: 2.4 GHz (Seendid Wifi 2G); ESP got IP 192.168.0.108

- Client: Python aiocoap PUT to coap://192.168.0.108/light

  - Two equivalent scripts were used; one also sets Content-Format: application/octet-stream.

3) Test Procedures & Results

A. Instruction ON (payload = b'1')

Client action

Message(code=PUT, uri="coap://192.168.0.108/light", payload=b"1")

ESP serial (observed – your screenshots):

[Light] Request received.

Payload received: 1

Instruction: Turn ON

Client console (observed):

ACK 2.04 Changed ...

Response Code: 2.04 Changed

Response Payload: 1

Result:  PASS — LED turns ON (LOW), server echoes "1".

B. Instruction OFF (payload = b'0')

Client action

Message(code=PUT, uri="coap://192.168.0.108/light", payload=b"0")

ESP serial (earlier run):

[Light] Request received.

Payload received: 0

Instruction: Turn OFF

Client console:

Response Code: 2.04 Changed

Response Payload: 0

Result:  PASS — LED turns OFF (HIGH), server echoes "0".

4) Observations

- The ESP prints the expected Instruction: Turn ON/OFF lines and mirrors the state in its CoAP response.

- Python side consistently receives 2.04 Changed with the correct payload ("1"/"0").

Arduino Output:(Instruction turned off)

Instruction turned on:

Vs output:

Situation:

payload = "0".encode('utf-8')

Situation:

payload = "1".encode('utf-8')